GEOG 3050

Lab:

Due:

Colorado River Basin Droughts

Water management in the American Southwest has been a hot topic for well over a century. Millions of people in the United States and Mexico rely on the Colorado River for water. In years where river flows are above average, there are generally few problems with water availability. However, extended periods of drought can pose a significant problem for water management, as seen historically during droughts in the American Southwest. To address this, the United States built numerous dams in the Colorado River Basin in order to store water during drought years. These allow storing excess water during high water years for use in low water years. Problem solved, right?

Following the completion of Hoover Dam (Lake Mead) in 1935 and Glen Canyon Dam (Lake Powell) in 1963, many thought problems with water availability in the West would become a thing of the past. These two reservoirs together hold roughly three times the annual flow of the Colorado River. Despite this truly massive storage capacity, the water levels for both of these reservoirs have been consistently declining for decades. Why? It turns out there just isn’t as much water as we thought there was.

The Colorado River Compact was signed in 1922 by seven states with the purpose of allocating the annual flow of the river among those states and Mexico. This compact relied on the accuracy of average annual flow estimates for the Colorado River at the time (~17 million acre-feet). Unfortunately, those flow estimates were based on data from what we now know as one of the wettest periods for the region in the last 450 years. As a result, significantly more water was allocated from the river than is actually present in an average year (likely between 13- and 14.5-million acre-feet). In addition to that, climate change is expected to further exacerbate the problem, both because of higher temperatures and more frequent periods of drought.

In this lab, we will look at how temperature and precipitation impact water availability in the Colorado River using two different approaches. This will require working with real-world data, which are commonly very noisy. Thus, we will also look at simple approaches for cleaning up and making use of noisy datasets to yield valuable information.

Before class:

1. Read:

Udall, B. and J. Overpeck (2017), The twenty-first century Colorado River hot drought and implications for the future, Water Resour. Res., 53, 2404– 2418, doi:10.1002/2016WR019638.

2. Define:

- Temperature Sensitivity (As defined in the paper)

- Precipitation Elasticity (As defined in the paper)

- Evapotranspiration

- bcm (units)

- Multiple linear regression

- Anomaly

- Water year

*Figures and Tables from Udall and Overpeck, 2017

Notes about our data:

In the paper, Udall and Overpeck examine temperature sensitivity and precipitation elasticity using two main data sources. The first is “natural flows” from Lee’s Ferry (a point on the Colorado River just below the Glen Canyon Dam) provided by the Bureau of Reclamation. These “natural flows” are measured flow volumes that were artificially corrected to estimate what the river’s natural flow would be if humans weren’t changing the system (through the use of dams, dam releases, agriculture, etc.). The second data source contains composite temperature and precipitation data from PRISM, which is a modeled, gridded source of climate data that is commonly used in many studies.

While we will be utilizing the same natural flows data used in the study, the temperature and precipitation data we will be using is different. Our temperature and precipitation data come from an automated weather station in Blanding, UT which means that they are point-source data measurements rather than composite, gridded, modeled estimates over the entire study region. Pay attention to these differences between their data and our data as you make calculations and compare our results to theirs.

Lab exercises:

1. Udall and Overpeck (2017) define two key terms in their study, temperature sensitivity and precipitation elasticity, which you have already defined above. Using these definitions, write equations for both of these terms, defining each variable used below the equation (including units).

Example:

The area of a circle is found by multiplying pi by the radius of the circle squared.

A=πr2

A = area of the circle (m2)

π = 3.14 (unitless constant)

r = radius of the circle (m)

2. In the study they examine the impact of precipitation and temperature anomalies on the flow of the Colorado River.

- Write an equation for calculating the precipitation anomaly and the temperature anomaly for a given year. Define each variable, including units.

- Why might they use the annual precipitation and temperature anomalies rather than just the annual precipitation and temperature data?

- Copy and paste the precipitation, temperature, and flow data from the table at the end of this document into Excel (or another appropriate program of your choice). In new columns, calculate the yearly temperature anomaly, precipitation anomaly, and flow anomaly compared to the reference period 1906-1999.

- Create 2 scatterplots: 1) Flow anomaly vs. Temperature anomaly, 2) Flow anomaly vs. Precipitation anomaly. Add trendlines to each, displaying both the trendline equations and their R2 values. How well does annual temperature and precipitation seem to predict annual flow values?

- Using these anomalies, recreate Table 2 from Udall and Overpeck (2017) using our data. How do our values compare to theirs?

- Add a row to the bottom of the table you just made for the period 2015-2021. Fill in all values for this new row.

3. In section 3 of the paper (Estimates of 2000–2014 Temperature-Induced Flow Loss) Udall and Overpeck give estimated ranges for temperature sensitivity and precipitation elasticity in the Upper Colorado River Basin that have previously been reported in other studies. Ranges for temperature sensitivity vary significantly, from -3.0%/°C to -13.8%/°C for temperature sensitivity and from 2 to 3 for precipitation elasticity. Here we will use the estimates they suggest of -6.5%/°C and 2.5.

- Using these estimates for temperature sensitivity and precipitation elasticity and Table 2 that you recreated with our data, create the table below and fill in all values.

| Period | d_F from T (%) | d_F from P (%) | Total Estimated d_F (%) | Actual d_F (%) |
| --- | --- | --- | --- | --- |
| 1953-1967 |  |  |  |  |
| 2000-2014 |  |  |  |  |
| 1906-1999 |  |  |  |  |
| 1906-2014 |  |  |  |  |
| 2015-2021 |  |  |  |  |

d_F from T (%) = Estimated percent change in flow attributable to temperature

d_F from P (%) = Estimated percent change in flow attributable to precipitation

Total Estimated d_F (%) = Estimated percent change in flow attributable to both temp and precip

Actual d_F (%) = Actual percent change in flow for the time period

- Compare columns Total d_F (%) and Actual d_F (%). How well do our estimates of change in flow compare to the actual changes in flow for these time periods?

4. In the last section, we assumed the estimates for temperature sensitivity and precipitation elasticity provided by Udall and Overpeck were accurate for our dataset, even though the temperature and precipitation data we are using is different from theirs. What if we wanted to analyze our data and come up with our own estimates? How can we examine the influence of two predictor variables (temperature and precipitation) on one independent variable (flow)? Here we will build on previous lessons that used regression to examine the relationship between a single predictor variable and a single independent variable. When we add additional predictor variables it is called multiple linear regression.

Before getting started in this section, performing multiple linear regression in Excel requires the add-in Analysis Toolpak. This may already be enabled, but before getting started you should check. To enable it, go to File>Options, and select the Add-ins tab on the left. At the bottom of the Add-ins tab is a drop-down menu that says Manage. Select Manage Excel Add-ins and press the Go button. This will bring up a list of the available Add-ins. Check the box for Analysis Toolpak and press OK.

- We’ll start by performing a multiple linear regression on our data. We will use temperature and precipitation anomalies as the predictor variables and inflow anomaly as the independent variable.

To do this in Excel, select the Data tab, then find the Analysis section on the far right, and select Data Analysis. (Note: The Data Analysis button will only be available if you have the Analysis Toolpak enabled as described above.) In the menu that pops up, select Regression and press OK. Select your input data for the regression. Remember, Y is your independent variable (flow anomaly) and X are your predictor variables (temperature and precipitation anomalies). Before you press OK, choose where you want Excel to put the results (either an output range of cells or a new worksheet ply).

- How well does this model explain the flow data? How can you tell?

- Oftentimes, real-world data are messy and require a bit of cleaning up to be very useful. While there are many ways to try to clean up messy data, one simple way is to smooth the data. Look at Figure 2, Panels b-d from Udall and Overpeck. Notice how their raw annual data are noisy and difficult to see a clear trend. However, their smoothed data (the blue lines) are much easier to make out trends. We will use this same idea to improve our multiple linear regression model.

We will be applying a 15-year moving average to smooth our data. There are multiple ways to do this, but an easy way is to use the Data Analysis tool that we used previously and select the Moving Average option (instead of Regression). You can also do this manually. Whichever method you choose, smooth your temperature anomaly, precipitation anomaly, and flow anomaly data into new columns.

- Repeat the steps in part 4a using your smoothed data. How well does this model explain flow data?

- Create a new column (Predicted Flow) and evaluate it using your 15-year average flow anomalies (i.e. given your 15-year averaged temperature and precipitation anomaly data, what does this model predict the flow anomaly should be?). Plot your 15-year average flow anomalies versus these new predicted values. (Hint: If your model was 100% perfect, each point in this plot would fall on the line y=x).

- Use the coefficient estimates from the regression model for temperature and precipitation to calculate new values for temperature sensitivity and precipitation elasticity. (Hint: The key here is paying close attention to the units of these coefficients and the units for each of the values you’re trying to calculate). Fill in the table below. How do our estimates for temperature sensitivity and precipitation elasticity compare to the ranges provided by Udall and Overpeck?

|  | Value | Units |
| --- | --- | --- |
| Temperature Coefficient |  |  |
| Precipitation Coefficient |  |  |
| Temperature Sensitivity |  |  |
| Precipitation Elasticity |  |  |

| Year | Mean Annual Temp (deg C) | Total Annual Precip (mm) | Natural Inflow (bcm) |
| --- | --- | --- | --- |
| 1906 | 10.4 | 579.1 | 23.1 |
| 1907 | 10.6 | 428.2 | 25.9 |
| 1908 | 9.0 | 460.8 | 14.4 |
| 1909 | 9.2 | 468.9 | 27.0 |
| 1910 | 10.4 | 327.7 | 18.0 |
| 1911 | 9.9 | 383.5 | 19.3 |
| 1912 | 8.4 | 373.4 | 23.0 |
| 1913 | 9.0 | 388.9 | 17.9 |
| 1914 | 10.1 | 469.1 | 26.0 |
| 1915 | 8.1 | 508.0 | 17.4 |
| 1916 | 8.3 | 446.0 | 23.7 |
| 1917 | 10.3 | 191.3 | 29.4 |
| 1918 | 9.1 | 283.0 | 19.4 |
| 1919 | 9.1 | 351.0 | 16.0 |
| 1920 | 9.1 | 329.4 | 27.1 |
| 1921 | 10.6 | 368.3 | 28.0 |
| 1922 | 8.9 | 319.3 | 22.9 |
| 1923 | 9.1 | 344.4 | 22.6 |
| 1924 | 9.5 | 247.9 | 18.1 |
| 1925 | 10.3 | 270.8 | 16.7 |
| 1926 | 12.1 | 390.4 | 20.1 |
| 1927 | 10.7 | 491.7 | 23.1 |
| 1928 | 9.3 | 367.5 | 21.9 |
| 1929 | 8.4 | 244.6 | 26.9 |
| 1930 | 8.8 | 322.3 | 18.7 |
| 1931 | 9.5 | 367.8 | 10.9 |
| 1932 | 8.4 | 284.2 | 22.0 |
| 1933 | 9.0 | 320.8 | 15.2 |
| 1934 | 11.6 | 179.6 | 8.1 |
| 1935 | 9.4 | 312.4 | 15.1 |
| 1936 | 10.3 | 300.7 | 17.9 |
| 1937 | 9.0 | 329.7 | 17.5 |
| 1938 | 10.0 | 374.9 | 22.1 |
| 1939 | 10.4 | 239.0 | 14.4 |
| 1940 | 10.6 | 482.6 | 11.6 |
| 1941 | 9.4 | 541.8 | 22.5 |
| 1942 | 9.4 | 197.4 | 23.9 |
| 1943 | 10.5 | 245.4 | 16.8 |
| 1944 | 9.1 | 299.2 | 19.0 |
| 1945 | 9.3 | 259.3 | 17.3 |
| 1946 | 10.2 | 289.8 | 13.6 |
| 1947 | 9.9 | 351.3 | 19.6 |
| 1948 | 9.1 | 348.5 | 19.4 |
| 1949 | 8.9 | 416.1 | 20.6 |
| 1950 | 10.4 | 135.1 | 16.2 |
| 1951 | 9.4 | 276.1 | 15.4 |
| 1952 | 9.3 | 290.6 | 25.7 |
| 1953 | 10.0 | 352.0 | 13.8 |
| 1954 | 11.0 | 284.5 | 10.2 |
| 1955 | 9.4 | 203.7 | 12.0 |
| 1956 | 10.3 | 125.2 | 14.3 |
| 1957 | 9.6 | 530.6 | 24.9 |
| 1958 | 11.0 | 237.2 | 20.8 |
| 1959 | 10.5 | 273.3 | 11.4 |
| 1960 | 10.2 | 197.6 | 14.8 |
| 1961 | 9.7 | 269.0 | 11.4 |
| 1962 | 10.4 | 289.8 | 21.9 |
| 1963 | 10.4 | 241.8 | 11.2 |
| 1964 | 9.5 | 197.1 | 12.8 |
| 1965 | 9.4 | 507.7 | 22.7 |
| 1966 | 10.2 | 279.4 | 13.6 |
| 1967 | 9.9 | 317.8 | 14.4 |
| 1968 | 9.2 | 301.5 | 16.5 |
| 1969 | 10.0 | 418.8 | 17.8 |
| 1970 | 9.8 | 282.7 | 18.6 |
| 1971 | 9.5 | 267.5 | 18.3 |
| 1972 | 10.2 | 363.0 | 15.3 |
| 1973 | 8.9 | 265.4 | 23.8 |
| 1974 | 9.7 | 314.7 | 16.0 |
| 1975 | 8.6 | 220.7 | 20.4 |
| 1976 | 9.9 | 156.7 | 13.8 |
| 1977 | 10.6 | 236.2 | 6.7 |
| 1978 | 10.3 | 492.8 | 18.4 |
| 1979 | 9.4 | 336.8 | 21.7 |
| 1980 | 10.7 | 331.5 | 21.4 |
| 1981 | 11.2 | 311.2 | 10.7 |
| 1982 | 9.5 | 373.9 | 20.6 |
| 1983 | 10.1 | 445.3 | 29.3 |
| 1984 | 10.1 | 284.2 | 29.8 |
| 1985 | 10.2 | 399.3 | 26.0 |
| 1986 | 10.8 | 390.9 | 27.6 |
| 1987 | 10.4 | 466.3 | 20.5 |
| 1988 | 10.5 | 289.6 | 14.4 |
| 1989 | 9.5 | 14.5 | 11.8 |
| 1990 | 11.3 | 340.1 | 11.1 |
| 1991 | 10.9 | 298.2 | 15.2 |
| 1992 | 11.3 | 382.0 | 13.6 |
| 1993 | 11.0 | 448.6 | 23.1 |
| 1994 | 12.2 | 373.6 | 13.1 |
| 1995 | 11.7 | 326.9 | 24.5 |
| 1996 | 12.4 | 315.5 | 17.3 |
| 1997 | 11.5 | 509.3 | 26.1 |
| 1998 | 12.0 | 361.2 | 20.9 |
| 1999 | 12.1 | 252.0 | 20.3 |
| 2000 | 12.4 | 375.7 | 13.0 |
| 2001 | 12.4 | 313.9 | 13.6 |
| 2002 | 12.2 | 200.4 | 7.2 |
| 2003 | 12.8 | 316.5 | 12.9 |
| 2004 | 11.8 | 390.9 | 11.6 |
| 2005 | 12.6 | 389.4 | 21.1 |
| 2006 | 12.2 | 299.2 | 15.6 |
| 2007 | 12.5 | 337.6 | 15.5 |
| 2008 | 11.6 | 429.3 | 20.1 |
| 2009 | 12.0 | 288.3 | 17.6 |
| 2010 | 11.5 | 426.7 | 15.2 |
| 2011 | 11.6 | 245.1 | 24.9 |
| 2012 | 12.4 | 207.5 | 10.4 |
| 2013 | 10.9 | 342.4 | 11.1 |
| 2014 | 12.2 | 230.4 | 17.4 |
| 2015 | 11.8 | 506.5 | 16.6 |
| 2016 | 11.8 | 265.7 | 16.6 |
| 2017 | 12.5 | 180.1 | 20.3 |
| 2018 | 12.3 | 215.9 | 10.5 |
| 2019 | 11.0 | 331.5 | 21.9 |
| 2020 | 12.2 | 145.3 | 11.9 |
| 2021 | 13.3 | 187.2 | 8.8 |

References

Udall, B. and J. Overpeck (2017), The twenty-ﬁrst centu ry Colorado River hot drought and implications for the future, Water Resour. Res., 53, 2404–2418, doi:10.1002/2016WR019638.